程序说明

本程序用于完成布匹表面缺陷检测任务，采用 Python 和 OpenCV 实现传统机器视觉检测，不使用神经网络方法。程序固定读取工作区中的正常样图 original.jpg 和待检测图 defect.jpg，对图像进行预处理、缺陷候选提取、区域筛选和结果绘制，最终输出检测结果图 result.jpg。

本程序的核心思想是利用正常样图作为参考，对缺陷图中的异常亮区域进行突出显示。主方法为“基于白帽变换的参考图差分法”：先分别对正常样图和缺陷样图进行灰度化、高斯滤波和白帽变换，以增强局部亮小目标；再将缺陷图的增强结果减去正常图的增强结果，从而抑制布匹重复纹理背景，突出划痕或破损等亮异常区域。之后通过阈值分割、形态学开闭运算、膨胀和去除贴边连通域等步骤，得到候选缺陷区域。

为了提高作业完成度，程序还实现了两种备选方法，分别是自适应阈值法和 Canny 边缘聚合法。当主方法未检测到可信缺陷时，程序会自动按顺序回退到备选方法继续检测，从而增强程序的鲁棒性和完整性。

在候选区域筛选阶段，程序使用连通域分析提取所有可能的缺陷区域，并综合区域面积、平均响应强度、最大响应强度以及长宽比进行评分，最终选择最符合缺陷特征的区域作为检测结果。这种方式可以减少噪声干扰，避免将普通纹理误判为缺陷。

程序最后会将检测到的缺陷区域以绿色框标出，并在结果图中标注所采用的方法名称，生成最终输出图 result.jpg。如果没有检测到可信缺陷，程序也会给出对应提示，保证运行过程完整且结果清晰。

流程图

结果图

思考题

思考题参考答案

1. 为什么本作业不能使用神经网络方法？

本作业要求使用传统机器视觉方法，主要有以下原因。首先，在工业检测场景中，很多任务的缺陷类型比较明确、成像条件相对固定，使用阈值分割、形态学处理、边缘检测等传统方法就可以较好地完成检测，没有必要引入训练成本更高的神经网络。其次，神经网络通常需要大量带标注的数据，而实际生产中缺陷样本往往较少，采集和标注成本较高。就算使用少量的带标注数据样本，也会导致检测出现严重的漏检，误检等问题。再次，传统方法计算量小、运行速度快、部署简单、结果更容易解释，更适合教学实验以及一些实时性要求较高的工业场景。因此，本作业限制不用神经网络，是为了训练对基础图像处理方法的理解和应用能力，和模拟真实工业应用环境

2. 你使用的检测方法有什么局限性？在什么情况下可能失效？

本程序采用的是“基于白帽变换的参考图差分法”，并辅以自适应阈值和 Canny 边缘检测作为备选方法。这类方法的局限性主要体现在以下几个方面。第一，它对成像条件较敏感，如果光照变化较大、相机位置偏移明显、图像发生旋转或缩放，正常样图和缺陷样图之间的差异会被放大，从而影响检测结果。第二，该方法更适合检测亮色、小尺度、局部突变型缺陷，如果缺陷是暗斑、低对比度磨损，或者与背景纹理非常相似，则可能漏检。第三，阈值和形态学参数依赖当前样图特征，当布匹纹理发生明显变化时，参数可能需要重新调整。第四，若正常样图本身存在噪声或污染，也可能导致误检。因此，该方法更适合成像环境稳定、缺陷类型相对固定的情况。

3. 如果要将算法部署到生产线，还需考虑哪些因素？

如果将该算法真正部署到生产线，还需要综合考虑工程上的多个因素。首先是成像稳定性，包括光源亮度、拍摄角度、相机位置、镜头参数和传送带速度等，这些因素直接影响检测效果。其次是实时性，需要保证算法能够在生产节拍内完成图像处理和结果输出。第三是鲁棒性，算法应尽量适应轻微的光照波动、布料抖动和纹理变化，减少误检和漏检。第四是系统集成问题，例如如何与 PLC、MES 或报警剔除系统联动。第五是维护和标定，包括定期清洁镜头、重新标定相机、更新阈值参数等。除此之外，还应考虑结果存档、缺陷统计、误报追踪和人工复核机制，便于后期质量管理和系统优化。

4. 传统机器视觉与深度学习各有什么优缺点？

传统机器视觉的优点是实现简单、运行速度快、对硬件要求较低、可解释性强，特别适合规则明确、背景稳定、缺陷特征清晰的场景。其缺点是对环境变化较敏感，泛化能力较弱，当缺陷类型复杂或样本变化较大时，往往需要人工调整参数。深度学习的优点是特征提取能力强，适合复杂背景、多类型缺陷和非规则目标检测，在大规模数据支持下通常能获得更高的准确率和更强的泛化能力。其缺点是需要大量训练数据和较高算力，训练与部署成本较高，可解释性相对较差。总体来看，传统机器视觉更适合小样本、规则明确、实时性要求高的任务；深度学习更适合复杂场景和大规模工业应用。

程序

如果可以请使用github，方便查看和拉取

本次作业项目地址: https://github.com/shixin08052/opencv-learing

detect.py

from __future__ import annotations

from pathlib import Path
from typing import Optional, Tuple

import cv2
import numpy as np


BBox = Tuple[int, int, int, int]


def load_images(base_dir: Path) -> Tuple[np.ndarray, np.ndarray]:
    original_path = base_dir / "original.jpg"
    defect_path = base_dir / "defect.jpg"

    original = cv2.imread(str(original_path), cv2.IMREAD_COLOR)
    defect = cv2.imread(str(defect_path), cv2.IMREAD_COLOR)

    if original is None:
        raise FileNotFoundError(f"Failed to read reference image: {original_path}")
    if defect is None:
        raise FileNotFoundError(f"Failed to read defect image: {defect_path}")
    if original.shape[:2] != defect.shape[:2]:
        raise ValueError(
            "Input images must have the same size, "
            f"got {original.shape[:2]} and {defect.shape[:2]}"
        )

    return original, defect


def preprocess_gray(image: np.ndarray) -> np.ndarray:
    gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)
    return cv2.GaussianBlur(gray, (5, 5), 0)


def remove_border_components(mask: np.ndarray) -> np.ndarray:
    num_labels, labels, stats, _ = cv2.connectedComponentsWithStats(mask, connectivity=8)
    cleaned = np.zeros_like(mask)
    height, width = mask.shape

    for label in range(1, num_labels):
        x, y, w, h, _ = stats[label]
        if x <= 0 or y <= 0 or x + w >= width or y + h >= height:
            continue
        cleaned[labels == label] = 255

    return cleaned


def component_score(
    area: int,
    aspect_ratio: float,
    mean_response: float,
    max_response: float,
) -> float:
    elongated_bonus = max(aspect_ratio - 1.2, 0.0) * 20.0
    return mean_response * 4.0 + area * 0.03 + max_response * 0.2 + elongated_bonus


def expand_bbox(
    bbox: BBox,
    image_shape: Tuple[int, int],
    pad_left: int = 0,
    pad_top: int = 0,
    pad_right: int = 0,
    pad_bottom: int = 0,
) -> BBox:
    x, y, w, h = bbox
    height, width = image_shape

    x1 = max(0, x - pad_left)
    y1 = max(0, y - pad_top)
    x2 = min(width, x + w + pad_right)
    y2 = min(height, y + h + pad_bottom)
    return x1, y1, x2 - x1, y2 - y1


def extract_best_component(
    mask: np.ndarray,
    score_map: Optional[np.ndarray] = None,
    min_area: int = 120,
) -> Tuple[np.ndarray, Optional[BBox], float]:
    binary = (mask > 0).astype(np.uint8) * 255
    num_labels, labels, stats, _ = cv2.connectedComponentsWithStats(binary, connectivity=8)

    best_mask = np.zeros_like(binary)
    best_bbox: Optional[BBox] = None
    best_score = 0.0

    for label in range(1, num_labels):
        x, y, w, h, area = stats[label]
        if area < min_area:
            continue

        component = (labels == label).astype(np.uint8) * 255
        contours, _ = cv2.findContours(component, cv2.RETR_EXTERNAL, cv2.CHAIN_APPROX_SIMPLE)
        if not contours:
            continue

        rect = cv2.minAreaRect(max(contours, key=cv2.contourArea))
        size_a, size_b = rect[1]
        short_side = max(min(size_a, size_b), 1.0)
        long_side = max(size_a, size_b)
        aspect_ratio = long_side / short_side

        if score_map is None:
            mean_response = float(area)
            max_response = float(area)
        else:
            values = score_map[labels == label]
            mean_response = float(values.mean())
            max_response = float(values.max())

        score = component_score(area, aspect_ratio, mean_response, max_response)
        if score <= best_score:
            continue

        best_score = score
        best_bbox = (int(x), int(y), int(w), int(h))
        best_mask = component

    return best_mask, best_bbox, best_score


def detect_by_reference_tophat(
    original: np.ndarray,
    defect: np.ndarray,
) -> Tuple[np.ndarray, Optional[BBox], float]:
    original_gray = preprocess_gray(original)
    defect_gray = preprocess_gray(defect)

    kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (19, 19))
    original_tophat = cv2.morphologyEx(original_gray, cv2.MORPH_TOPHAT, kernel)
    defect_tophat = cv2.morphologyEx(defect_gray, cv2.MORPH_TOPHAT, kernel)
    score_map = cv2.subtract(defect_tophat, original_tophat)
    score_map = cv2.GaussianBlur(score_map, (5, 5), 0)

    threshold_value, binary = cv2.threshold(
        score_map, 0, 255, cv2.THRESH_BINARY + cv2.THRESH_OTSU
    )
    if threshold_value < 13:
        _, binary = cv2.threshold(score_map, 13, 255, cv2.THRESH_BINARY)

    open_kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5))
    close_kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (7, 7))
    binary = cv2.morphologyEx(binary, cv2.MORPH_OPEN, open_kernel)
    binary = cv2.morphologyEx(binary, cv2.MORPH_CLOSE, close_kernel)
    binary = cv2.dilate(binary, open_kernel, iterations=1)
    binary = remove_border_components(binary)

    return extract_best_component(binary, score_map=score_map, min_area=150)


def detect_by_adaptive(defect: np.ndarray) -> Tuple[np.ndarray, Optional[BBox], float]:
    gray = preprocess_gray(defect)
    clahe = cv2.createCLAHE(clipLimit=2.0, tileGridSize=(8, 8))
    enhanced = clahe.apply(gray)
    score_map = cv2.morphologyEx(
        enhanced,
        cv2.MORPH_TOPHAT,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (15, 15)),
    )

    binary = cv2.adaptiveThreshold(
        score_map,
        255,
        cv2.ADAPTIVE_THRESH_GAUSSIAN_C,
        cv2.THRESH_BINARY,
        35,
        -4,
    )
    binary = cv2.morphologyEx(
        binary,
        cv2.MORPH_OPEN,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (3, 3)),
    )
    binary = cv2.morphologyEx(
        binary,
        cv2.MORPH_CLOSE,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5)),
    )
    binary = remove_border_components(binary)

    return extract_best_component(binary, score_map=score_map, min_area=120)


def detect_by_canny(defect: np.ndarray) -> Tuple[np.ndarray, Optional[BBox], float]:
    gray = preprocess_gray(defect)
    score_map = cv2.morphologyEx(
        gray,
        cv2.MORPH_TOPHAT,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (15, 15)),
    )

    edges = cv2.Canny(score_map, 40, 120)
    link_kernel = cv2.getStructuringElement(cv2.MORPH_RECT, (3, 3))
    edges = cv2.dilate(edges, link_kernel, iterations=1)
    edges = cv2.morphologyEx(edges, cv2.MORPH_CLOSE, link_kernel, iterations=2)
    binary = cv2.morphologyEx(
        edges,
        cv2.MORPH_OPEN,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (3, 3)),
    )
    binary = remove_border_components(binary)

    return extract_best_component(binary, score_map=score_map, min_area=120)


def draw_detection(
    defect_bgr: np.ndarray,
    mask: np.ndarray,
    bbox: Optional[BBox],
    method_name: str,
) -> np.ndarray:
    result = defect_bgr.copy()
    overlay = result.copy()
    display_mask = cv2.dilate(
        mask,
        cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5)),
        iterations=1,
    )
    overlay[display_mask > 0] = (0, 255, 0)
    result = cv2.addWeighted(overlay, 0.25, result, 0.75, 0)

    if bbox is not None:
        x, y, w, h = expand_bbox(
            bbox,
            defect_bgr.shape[:2],
            pad_left=4,
            pad_top=10,
            pad_right=4,
            pad_bottom=4,
        )
        cv2.rectangle(result, (x, y), (x + w, y + h), (0, 255, 0), 2)

    cv2.putText(
        result,
        f"Method: {method_name}",
        (12, 28),
        cv2.FONT_HERSHEY_SIMPLEX,
        0.75,
        (0, 255, 0),
        2,
        cv2.LINE_AA,
    )
    return result


def select_detection(original: np.ndarray, defect: np.ndarray) -> Tuple[np.ndarray, Optional[BBox], str]:
    methods = [
        ("reference_tophat", lambda: detect_by_reference_tophat(original, defect)),
        ("adaptive_threshold", lambda: detect_by_adaptive(defect)),
        ("canny_merge", lambda: detect_by_canny(defect)),
    ]

    for method_name, method in methods:
        mask, bbox, score = method()
        if bbox is not None and score > 0:
            return mask, bbox, method_name

    empty_mask = np.zeros(defect.shape[:2], dtype=np.uint8)
    return empty_mask, None, "none"


def main() -> None:
    base_dir = Path(__file__).resolve().parent
    output_path = base_dir / "result.jpg"

    original, defect = load_images(base_dir)
    mask, bbox, method_name = select_detection(original, defect)

    result = draw_detection(defect, mask, bbox, method_name)
    if bbox is None:
        cv2.putText(
            result,
            "No confident defect found",
            (12, 56),
            cv2.FONT_HERSHEY_SIMPLEX,
            0.7,
            (0, 255, 255),
            2,
            cv2.LINE_AA,
        )

    if not cv2.imwrite(str(output_path), result):
        raise IOError(f"Failed to save result image: {output_path}")

    if bbox is None:
        print(f"Saved result to: {output_path}")
        print("No confident defect was detected.")
        return

    x, y, w, h = bbox
    print(f"Saved result to: {output_path}")
    print(f"Selected method: {method_name}")
    print(f"Defect bbox: x={x}, y={y}, w={w}, h={h}")


if __name__ == "__main__":
    main()